Basic Arduino Functions

Quick Reference Guide for Beginners

This document introduces the most commonly used Arduino functions for beginners. Examples are written in Arduino C/C++ style and can be used with boards such as the Arduino Uno.

# 1. Arduino Program Structure

Every Arduino sketch normally contains setup() and loop(). setup() runs once when the board starts or resets. loop() runs repeatedly.

void setup() {
  // Initialization code
}

void loop() {
  // Code that repeats
}

# 2. pinMode()

Used to configure a digital pin as INPUT, OUTPUT, or INPUT_PULLUP.

pinMode(pin, mode);

// Example
pinMode(13, OUTPUT);
pinMode(2, INPUT);
pinMode(4, INPUT_PULLUP);

# 3. digitalWrite()

Sets a digital output pin HIGH (approximately 5 V on a typical Uno) or LOW (0 V).

digitalWrite(pin, value);

// Example
digitalWrite(13, HIGH);
digitalWrite(13, LOW);

# 4. digitalRead()

Reads the logic level of a digital input pin. It returns HIGH or LOW.

int state = digitalRead(2);

if (state == HIGH) {
  // Input is HIGH
}

# 5. analogRead()

Reads an analog input. On a typical Arduino Uno, analogRead() returns a value from 0 to 1023 for an input voltage from 0 V to the reference voltage (normally about 5 V).

int value = analogRead(A0);
Serial.println(value);

# 6. analogWrite()

Outputs a PWM signal on supported PWM pins. The value is normally 0 to 255: 0 = 0% duty cycle, 255 = 100% duty cycle. It is commonly used for LED brightness or motor control.

analogWrite(9, 128);   // approximately 50% duty cycle
analogWrite(9, 255);   // 100%
analogWrite(9, 0);     // 0%

# 7. delay()

Pauses the program for the specified number of milliseconds.

delay(1000);   // 1 second
delay(500);    // 0.5 second

Note: delay() blocks the processor from executing the next statements during the delay.

# 8. millis()

Returns the number of milliseconds since the Arduino program started. It is useful for timing without blocking the program with delay().

unsigned long startTime = millis();

# 9. micros()

Returns the number of microseconds since the Arduino program started. It is useful for short timing measurements.

unsigned long t = micros();

# 10. Serial Communication

Serial functions are useful for debugging and communicating with a computer or another serial device.

void setup() {
  Serial.begin(9600);
}

void loop() {
  Serial.println("Hello Arduino");
  delay(1000);
}

Common Serial functions:

Serial.begin(baudrate) – starts serial communication.

Serial.print() – prints data without moving to a new line.

Serial.println() – prints data and moves to a new line.

Serial.read() – reads incoming serial data.

Serial.available() – checks whether received data is available.

# 11. Basic Data Types

| Type | Example | Use |
| --- | --- | --- |
| int | int count = 10; | Integer values |
| float | float temp = 25.5; | Decimal values |
| char | char c = 'A'; | Single character |
| bool | bool flag = true; | true/false |
| unsigned long | unsigned long t = millis(); | Timing values |

# 12. Constants Commonly Used

HIGH       // Logic HIGH
LOW        // Logic LOW
INPUT      // Digital input
OUTPUT     // Digital output
INPUT_PULLUP // Input with internal pull-up resistor

# 13. A Simple LED Blink Example

This example demonstrates pinMode(), digitalWrite(), and delay().

const int LED_PIN = 13;

void setup() {
  pinMode(LED_PIN, OUTPUT);
}

void loop() {
  digitalWrite(LED_PIN, HIGH);
  delay(1000);

  digitalWrite(LED_PIN, LOW);
  delay(1000);
}

# 14. Button + LED Example

The LED turns ON when the button is pressed. INPUT_PULLUP is used, so the input reads LOW when the button is pressed.

const int LED_PIN = 13;
const int BUTTON_PIN = 2;

void setup() {
  pinMode(LED_PIN, OUTPUT);
  pinMode(BUTTON_PIN, INPUT_PULLUP);
}

void loop() {
  int buttonState = digitalRead(BUTTON_PIN);

  if (buttonState == LOW) {
    digitalWrite(LED_PIN, HIGH);
  } else {
    digitalWrite(LED_PIN, LOW);
  }
}

# 15. Analog Sensor Example

Reads a potentiometer connected to A0 and displays the raw ADC value on the Serial Monitor.

const int SENSOR_PIN = A0;

void setup() {
  Serial.begin(9600);
}

void loop() {
  int sensorValue = analogRead(SENSOR_PIN);
  Serial.println(sensorValue);
  delay(200);
}

# 16. Quick Function Reference

| Function | Purpose | Example |
| --- | --- | --- |
| setup() | Runs once at startup | void setup() {} |
| loop() | Runs repeatedly | void loop() {} |
| pinMode() | Configures a pin | pinMode(13, OUTPUT); |
| digitalWrite() | Sets digital output | digitalWrite(13, HIGH); |
| digitalRead() | Reads digital input | digitalRead(2); |
| analogRead() | Reads analog input | analogRead(A0); |
| analogWrite() | Generates PWM | analogWrite(9, 128); |
| delay() | Blocking delay in ms | delay(1000); |
| millis() | Elapsed time in ms | millis(); |
| Serial.begin() | Starts serial | Serial.begin(9600); |
| Serial.println() | Prints a line | Serial.println(value); |

# 17. Important Notes for Students

Always configure a digital pin with pinMode() before using it as an input or output.

Do not connect an LED directly to a GPIO pin without an appropriate current-limiting resistor.

Check the board's pinout before using PWM, analog, or special-function pins.

analogWrite() is PWM output on boards such as the Uno; it is not a true analog voltage output.

Use millis() when you need multiple tasks to run without blocking the program with delay().

Use Serial Monitor frequently to debug sensor readings and program flow.